|  | University of Hertfordshire - Individual Project Risk Assessment Form |  |
| --- | --- | --- |

| Project Title: Natural-Language Control of a Sub-250g Low-Cost Autonomous Drone | Faculty: EE / ME [circle one — confirm with supervisor] | Batch No: [insert] | Full Time / Part Time [circle one] |
| --- | --- | --- | --- |
| Student Name: [insert] | Student No: [insert] |  | Contact No: [insert] |

| SECTION B: RISK ASSESSMENT |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. HAZARD IDENTIFICATION |  |  |  | 2. RISK EVALUATION |  |  |  | 3. RISK CONTROL |  |  |  |
| No. | Work Activity | Hazard | Possible Injury /Ill-Health | Existing Risk Controls | Severity | Likelihood | RPN | Additional Controls | Severity | Likelihood | New RPN |
| 1 | Flight testing – bench motor spin-up and manual/autonomous flight | Rotating propellers | Laceration to hands, face or eyes | Propeller guards fitted for all flight testing; propellers removed during bench and configuration work; motors armed only in a cleared area; eye protection worn | 3 | 3 | 9 | Kill-switch (software disarm) enforced at all times; only project team may power the aircraft; safety briefing given before each test session | 2 | 2 | 4 |
| 2 | Charging, storage and post-crash inspection of batteries | LiPo battery fire / thermal runaway | Burns, smoke inhalation, fire damage | Charged only in a LiPo-safe fireproof bag; never charged unattended; stored at storage voltage | 4 | 2 | 8 | Visual inspection after every crash; swollen or damaged cells discarded immediately via e-waste channel; fire extinguisher / fire-safe container located in lab | 3 | 1 | 3 |
| 3 | Manual and autonomous flight testing indoors | Uncommanded flight / loss of control (fly-away) | Impact injury to persons; property damage | Indoor netted or enclosed testing area; tethered and low-altitude first flights; software kill-switch | 3 | 3 | 9 | Second person present as spotter; testing restricted to staffed laboratory hours; incremental expansion of altitude/speed envelope | 2 | 2 | 4 |
| 4 | PCB and wiring assembly / rework | Hot soldering iron and flux fumes | Burns; irritation from fume inhalation | Laboratory induction required before use; fume extraction in place; iron returned to stand when not in use | 2 | 3 | 6 | Safety glasses worn; iron unplugged when unattended for an extended period | 1 | 2 | 2 |

| Student's Name & Signature: |  | Sup's Name & Signature: |  | Lab Eng’s Name & Signature: |  |
| --- | --- | --- | --- | --- | --- |
| Date: |  | Date: |  |  |  |